X680 DOCUMENTATION FILE

CREATION DATE: 1-3-71

LAST UPDATE: 4-1-71

FILE NAME: X68DOC.RNO

THE INFORMATION IN THIS DOCUMENT IS SUBJECT  TO

CHANGE   WITHOUT   NOTICE  AND  SHOULD  NOT  BE

CONSTRUED AS A COMMITMENT BY DIGITAL  EQUIPMENT

CORPORATION.

X680 DOCUMENTATION FILE                             PAGE   2

1.0  CHANGES

VERSION-1 UPDATE-A ---------

1.   THE RESTART PROCEEDURE HAS BEEN CHANGED:

IF FOR ANY REASON X680 NEEDS TO BE  INITIALIZED  (E.G.,

AFTER  A HALT) WITH ACTIVE USERS CONNECTED, PERFORM THE

FOLLOWING STEPS:

1.  PRESS STOP TO STOP THE 680I.

2.  PUT 0001 IN THE SWITCHES AND LOAD ADDRESS

3.  PUT 5405 IN THE SWITCHES AND DEPOSIT

4.  RESET THE SWITCHES TO DESIRED STARTING ADDRESS  AND

CONTINUE

NOTE -- USERS OF P680.PAL VERSIONS BEFORE  UPDATE-A

WILL  ALSO  HAVE  TO  DEPOSIT  200 IN LOCATION 0005

AFTER THE DEPOSIT IN LOCATION 0001.

2.0  DEFICIENCIES

VERSION-1 UPDATE-A -------

1.   IN THE CONFIGURATION  FILE  (SEE  680DES.RNO),  DO  NOT

SPECIFY  MODEM  CONTROL  (MODCON=1) UNLESS YOU ACTUALLY

INCLUDE SOME LINES WITH MODEM CONTROL (REMOTE OR MODEM)

IN THE LINES ELEMENT.

X680 DOCUMENTATION FILE                             PAGE   3

3.0  COMPATIBILITY REQUIREMENTS

1.  M680.MAC AND CONFIGURATION FILE MUST  BE  ASSEMBLED

WITH MACRO V.44 AND LOADED WITH LOADER V.52.

2.   ALWAYS  USE  IMAGE  MODE  (PIP  SWITCH  /I)   WHEN

TRANSFERING .BIN OR .RIM FILES (E.G.  L680.RIM).

3.0 X680 WILL RUN WITH ALL PDP-10 MONITORS  SUBJECT  TO

THE FOLLOWING CONSIDERATIONS:

X680A     L680

4S72    (1) (2)   (3)

5S01    (1) (2)   (3)

5S02    (2)       --

5S03    --        --

(1)  THESE  MONITORS  DO  NOT   SUPPORT   DATA-SET

CONTROL, THEREFORE IF YOU HAVE DATA-SETS, USE

THE  CONSOLE  TTY  TO  ENABLE  ALL  DATA-SETS

(7777A)  AND  RUN WITH MODEM CONTROL DISABLED

(SWITCH-10 = 1).  NOTE--  THERE  WILL  BE  NO

AUTO-DETACH  ON  DATA-SET  DISCONNECTS  SINCE

THIS  IS  AGAIN  A  FUNCTION  OF  THE  PDP-10

MONITOR.

(2)  RESTARTING OR RELOADING  THE  PDP-10  MONITOR

DOES NOT RESTART X680 WITH THESE MONITORS.

(3)  THESE  MONITORS  DO  NOT  SUPPORT  IMAGE-MODE

TELETYPE  OUTPUT  AND THEREFORE THE PDP-10 TO

680I LOADER MODE WILL NOT WORK.   THE  ASR-33

MODE  WILL  STILL  WORK BUT IT IS THE SAME AS

THE STANDARD BINARY LOADER.