THE MAJOR DEFICIENCY IN THE 5.03 REMOTE PACKAGE IS THE LACK

OF THE REMOTE LOAD AND START PROCEDURE DESCRIBED IN THE USER'S MANUAL

(DEC-10-NUZA-D) SECTIONS 2.2.2 AND 2.2.3.  THIS IN TURN LEADS TO A

SECURITY DEFICIENCY, AS FOLLOWS:

THE CODE TO CHECK FOR "NOT IN CONTACT" IS PERMANENTLY

DISABLED, SO THAT OUTPUT CAN BE DIRECTED TO A REMOTE

PERIPHERAL AT ANY TIME.

THERE IS NO PROVISION FOR STATION IDENTIFICATION ON

CALLUP.

THUS, IF CONTACT IS LOST ON A DIAL UP LINE AND A

DIFFERENT STATION CALLS IN, ITS PERIPHERALS AND TTY CAN

RECEIVE MESSAGES INTENDED FOR THE STATION PREVIOUSLY

IN CONTACT.  EVEN IF THERE WAS NO PREVIOUS CONTACT,

MESSAGES FOR BATCH DEVICES CAN BE MISDIRECTED.

THE CODING FOR THE AUTOMATIC START UP IS IN THE MONITOR, BUT

IS NOT COMPLETELY DEBUGGED.  IT SHOULD BE AVAILABLE IN THE NEAR

FUTURE.